SECTION 102819

GLASS SHOWER DOORS

- GENERAL

      - DESCRIPTION OF WORK

        - Work Included:  Provide labor, materials and equipment necessary to complete the work of this Section including the following.

          - All-glass swinging shower door assemblies.

        - Related Work:  The following items are not included in this Section and are specified under the designated Sections:

          - Section 061000 - Rough Carpentry for blocking in walls.

      - SUBMITTALS

        - Product Data:  Submit for each product indicating materials, finish, dimensions, and profiles.  Include installation instructions.

        - Shop Drawings:  Submit shop drawings indicating plans, elevations, details of construction, and relationship with adjacent construction.

      - QUALITY ASSURANCE

        - Mock-Up:  Provide an in-place mock-up of first installation for each type for review of workmanship.  Accepted mock-ups may remain in place.

      - DELIVERY, STORAGE, AND HANDLING

        - Delivery:  Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

        - Storage and Handling:  Comply with manufacturer's recommendations for storage and handling.  Protect from weather damage.

      - WARRANTY

        - Warranty:  Provide manufacturer's standard limited warranty against defects in manufacturing.

- PRODUCTS

      - MANUFACTURER

        - Basis-of-Design for Glass Shower Doors:   Luca + Lex, www.luca-lex.com, tel 978-296-5578.

          - Shower Door Style:  Encompass.

          - Shower Door Style:  Infinity.

        - Tempered Float Glass:  ASTM C 1048; Type I (transparent flat glass); Quality-Q3; Kind FT.

          - Door Glass:  3/8 inch clear tempered glass.

          - Panel Glass:  3/8 inch clear tempered glass.

        - Metal Finish:  [Polished chrome.] [Brushed nickel.] [Matte black.] [Brushed bronze.]

        - Glass-To-Wall Hinges: Manufacturers standard adjustable solid brass, 90 degrees swing both directions, suitable for width of glass.  Self-centering and closing from 15 degrees.

          - Hinge Quantity:  Provide 2 hinges for doors maximum 80 pounds, maximum 28 inch door width.  Provide 3 hinges for doors maximum 120 pounds , maximum 32 inch door width.

          - Hinge Gaskets:  One thick gasket on each side of glass.

          - Assembly:  Provide door glass with hinges and hinge rail pre-assembled.

        - Accessories:

          - Handle Style:  10 inch overall back-to-back Biscayne/ladder pull fabricated from hollow brass tube 3/4 inch OD with 1/2 inch holes at 6 inches on center.  Include washers, insert and allen wrench.

          - Handle Style:  6 inch back-to-back Acadia/C-pull fabricated from hollow brass tube 3/4 inch OD with 1/2 inch holes at 6 inches on center.  Include washers, insert and allen wrench.

          - Wall Attachment:  U-channels  3/32 inch aluminum wall thickness.

          - Wall Attachment:  Wall-mount clamp 3/32 inch aluminum wall thickness.

          - Adjustable Jambs, Optional for Encompass Style Only:  Adjustable aluminum hinge jambs and wall jambs, 1/8 inch wall thickness.

          - Door Sweep:  Polycarbonate sweep.

          - Bottom Wipe:  Polycarbonate with drip edge.

          - Fasteners:  Stainless steel, type as recommended by manufacturer based on substrate.

- EXECUTION

      - EXAMINATION

        - Examine existing conditions to determine that they are suitable for installation.  Proceed with installation only when unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Install units in accordance with manufacturer's instructions, approved submittals, and in proper relationship with adjacent construction.

        - Do not use power tools to install hinges.

      - ADJUSTING AND CLEANING

        - Adjust units for proper operation, clearance and uniform appearance.  Do not use abrasive cleaners on hinges.  Clean surfaces using materials acceptable to manufacturer.

END OF SECTION